Shen Zhen Union Security Industry Co., Ltd

GPRS Communication Protocol Between GPS Tracker and Server

version V2.02

T360-101A/101E/103/269/108

| Change Version | Change Record | Date |
| --- | --- | --- |
| V1.00 | Build file | 2008-8-21 by George |
| V1.01 | Add parking overtime with time | 2008-10-23 By George |
| V1.02 | Change sign in to illegal ignition and add illegal ignition alarm | 2008-10-23 By George |
| V1.10 | Add user define alarm bit in alarm info | 2008-12-16 By George |
| V1.20 | Change fatigue drive alarm mode, add clear value for illegal ignition alarm , add penetrate transmit data for serial 2-7 | 2009-3-16 By George |
| V1.28 | Add acc status in 0x82 packet | 2009-4-30 By JGShi |
| V1.41 | Add remote update result upload A5 command. | 209-7-21 By George |
| V2.00 | Format change and add protocol for new functions in 108 | 2012.5-15 By liven |
| V2.01 | Add 8E data description | 2012.12-19 By Liven |
| V2.02 | Add command 0x71, 0xA3, 0xD3,0xD4,0xD9,0xDA, 0xDB, 0xDC, Appendix1 Position Data. | 2013.06-15 By Wayne |

Content

I Command Format	4

II Command List	5

III Command Details	6

1.	Login – 0xB1	6

2.	Login/Server Confirmation – 0x21	6

3.	One time track on demand – 0x30	6

4.	One time track on demand reply – 0x81	7

5.	Tracking by Interval (Acc_On) – 0x34	7

6.	Tracking by Interval (Acc_Off) – 0x70	7

7.	Track by Distance – 0x35	8

8.	Position Data Upload (by Interval, distance) – 0x80	8

9.	Blind Area Data re-Upload – 0x8E	8

10.	Over speed Alarm Set – 0x3F	9

11.	Movement Alarm – 0xC7	9

12.	Stop Engine (Remote Immobilize) – 0x39	9

13.	Recover Engine (Disable Immobilize) – 0x38	9

14.	Listen in (Voice Monitoring) – 0x3E	10

15.	Set Geo-Fencing – 0x46	10

16.	Cancel Geo-Fencing – 0x47	11

17.	Inquiry GEO Fencing – 0x48	11

18.	Send Dispatch Info – 0x3A	11

19.	Delete Mileage – 0x66	12

20.	Alarm Info – 0x82	12

21.	Cancel Alarm – 0x37	12

22.	Set GPRS Parameters – 0xC1	13

23.	Set Server IP and Port In Device – 0x69	13

24.	Set Reply – 0x85	13

25.	Illegal Ignition Set -0xC6	14

26.	Device Factory Reset – 0xC3	14

27.	Set Vehicle Idle Alarm – 0x4B	15

28.	Check Device Version – 0x3D	15

29.	Check Device Version Reply – 0x84	15

30.	Get Current Address – 0xA8	16

31.	Get Current Address Reply – 0x72	16

32.	Remote Update Firmware – 0xD2	16

33.	Remote Update Firmware Result – 0xA5	17

34.	Camera Related Command – 0x28/0x65/0x26/0xAB	17

35.	Temperature alarm set – 0x71	17

36.	Peripheral data info upload –0xA3	18

37.	Fuel level sensor Parameter Set – 0xD3	18

38.	Real-time tracking by Interval – 0xD4	19

39.	Tracking compensation by angle – 0xD9	19

40.	Fatigue driving alarm set – 0xDA	20

41.	Over Speed alarm and trig-time set – 0xDB	20

42.	Device odometer value set – 0xDC	21

Appendix 1 Position Data	22

Appendix 2 Alarm Data	25

Appendix 3 peripheral data	26

# I Command Format

Commands format of GPRS Packets are defined as following:

From Server to Device:

))<Command(1byte)><L(2 bytes)><ID(4bytes)><Parameter(s)><Checksum>\r

From Device to Server:

))<Command(1byte)><L(2 bytes)><ID(4bytes)><data><Checksum>\r

Note:

- Do not input ‘<’ and ‘>’ in the command content.

- All Muti-byte data complies with the following sequence: High byte prior to the low byte (Big Endian).

- The size of one location data packet is about 50 bytes.

| Item | Specification |
| --- | --- |
| )) | 2bytes. It is header for all correct packets. ‘))’ is ASCII code, in hex code it is 0x29 0x29. |
| Command | 1 byte. It is in hex code, every packet have one and only command word. For detail, please refer to Part II Command List |
| L | 2 bytes. It means the length from Length byte (not include length byte) to packet end, include the end byte. It is in hex code. |
| ID | 4bytes. It is in hex code. Every device has one and only ID, It is calculate from the device SN. For example: Suppose the device SN is: 29800298 Divide into 4 parts: 29 80 02 98 Convert to hex: 1D 50 02 62 Add 0x80 to 2nd,3rd part: 1D D0 82 62 Then you get the device ID: 1DD08262 |
| Parameter/data | Minimum 0 byte, maxmum 200byte, see appendix 1 for data description. |
| Checksum | 1 bytes, it is xor checksum, for example: if the packet is: 29 29 B1 00 07 0A 9F 95 38 0C 82 0D 0x82= 29 xor 29 xor B1 xor 00 xor 07 xor 0A xor 9F xor 95 xor 38 xor 0C |
| \r | 1 bytes, it is end byte of package, in hex it is 0D |

# II Command List

Command                                                         Page

1.	Login – 0xB1	6

2.	Login/Server Confirmation – 0x21	6

3.	Track on Demand – 0x30	6

4.	Track on Demand Reply – 0x81	7

5.	Track by Interval (motion) – 0x34	7

6.	Track by Interval (motionless) – 0x70	7

7.	Track by Distance – 0x35	8

8.	Position Data Upload (by Interval, distance) – 0x80	8

9.	Blind Area Data re-Upload – 0x8E	8

10.	Over speed Alarm Set – 0x3F	9

11.	Movement Alarm – 0xC7	9

12.	Stop Engine (Remote Immobilize) – 0x39	9

13.	Recover Engine (Disable Immobilize) – 0x38	9

14.	Listen in (Voice Monitoring) – 0x3E	10

15.	Set Geo-Fencing – 0x46	10

16.	Cancel Geo-Fencing – 0x47	11

17.	Inquiry GEO Fencing – 0x48	11

18.	Send Dispatch Info – 0x3A	11

19.	Delete Mileage – 0x66	12

20.	Alarm Info – 0x82	12

21.	Cancel Alarm – 0x37	12

22.	Set GPRS Parameters – 0xC1	13

23.	Set Server IP and Port In Device – 0x69	13

24.	Set Reply – 0x85	13

25.	Illegal Ignition Set -0xC6	14

26.	Device Factory Reset – 0xC3	14

27.	Set Vehicle Idle Alarm – 0x4B	15

28.	Check Device Version – 0x3D	15

29.	Check Device Version Reply – 0x84	15

30.	Get Current Address – 0xA8	16

31.	Get Current Address Reply – 0x72	16

32.	Remote Update Firmware – 0xD2	16

33.	Remote Update Firmware Result – 0xA5	17

34.	Camera Related Command – 0x28/0x65/0x26/0xAB	17

# III Command Details

## Login – 0xB1

| Command: | ))<0xB1><L><ID><reserved><Checksum><\r> |
| --- | --- |
| Description: | After your device installed properly, device will send this packet to server for login. It will send every 30s, like heart beat.It must be reply, If server does not give reply (see Login confirmation), device will keep send this packet without any other packet; after get reply, other commands start work. |
| Example: | 29 29 B1 00 07 0A 9F 95 38 0C 82 0D |
| Note: | In example, the ID is 0A9F9538, change to device SN, it is 10312156. In <content> part 0x0C, we keep it as future use; do not have certain meaning now. |

## Login/Server Confirmation – 0x21

| Command: | ))<0x21><L><receive package check sum><receive package command><reserved><Checksum><\r> |
| --- | --- |
| Description: | This command is send from server to device, to confirm some information, like login, some important alarm, and so on. |
| Example: | 29 29 21 00 05 82 B1 0C 1B 0D |
| Note: | The example is the confirmation for packet 29 29 B1 00 07 0A 9F 95 38 0C 82 0D, <receive package check sum> is 0x82, <receive package command> is 0xB1,<receive package sub-command(first byte of the content)> part is 0x0C. Example 2(if device need server to give 0x21 ack): 29 29 82 00 23 0C 80 80 32 14 10 24 15 10 58 02 23 57 72 11 35 28 35 00 00 00 00 FB 00 00 00 01 00 00 00 00 00 00 25 0D The reply should be: 29 29 21 00 05 25 82 14 97 0D. |

## One time track on demand – 0x30

| Command: | ))<0x30><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command will get current location data when you send, and device only upload location data once. About the device reply, See Reply. |
| Example: | 29 29 30 00 06 0A 9F 95 38 0E 0D |
| Note: | This command used when you want track manually, you can send this command from software platform to device. |

## One time track on demand reply – 0x81

| Command: | ))<0x81><L><ID><Position data><Checksum><\r> |
| --- | --- |
| Description: | Reply packet for track on demand command; send current position, speed, and status data to server. From device to server. About <Position data> description , please see Appendix 1 Positon data |
| Example: | 29298100280A9F9538081228160131022394301140372500000330FF0000007FFC0F00001E000000000034290D |
| Note: | The data between <ID> and <Checksum> is Position data. |

## Tracking by Interval (Acc_On) – 0x34

| Command: | ))<0x34><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This is command for setting device upload by time interval when ACC on. Our device default upload interval is 30s, you can change or set it with this command, the content of <parameter> part is interval, and unit is second, in hex code. After receive this command, device will give Set Reply. And the packet send by interval to server see Position Data Upload. |
| Example: | 29 29 34 00 08 0A 9F 95 38 00 1E 1A 0D |
| Note: | In this example, the interval is 001E, In decimal it is 30, so this command will set the device upload with interval 30s. |

## Tracking by Interval (Acc_Off) – 0x70

| Command: | ))<0x70><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This is command for setting device upload by time interval when ACC off. Our device default upload interval is 30s, you can change or set it with this command, the content of <parameter> part is interval, and unit is second, in hex code. After receive this command, device will give Set Reply. And the packet send by interval to server see Position Data Upload. |
| Example: | 29 29 70 00 08 0A 9F 95 38 00 1E 5E 0D |
| Note: | In this example, the interval is 001E, In decimal it is 30, so this command will set the device upload with interval 30s. |

## Track by Distance – 0x35

| Command: | ))<0x35><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command set device upload when moving a certain distance. In default device have close this function, use this command to change the upload rule to distance interval. The content of <parameter> part is distance interval; the unit of it is meter, in hex code. After receive this command device will send Set Reply, And the packet send by interval to server see Position Data Upload. |
| Example: | 29 29 35 00 08 0A 9F 95 38 00 64 61 0D |
| Note: | In this example, the interval is 0064, in decimal it is 100, the device will upload with interval 100m. |

## Position Data Upload (by Interval, distance) – 0x80

| Command: | ))<0x80><L><ID><data><Checksum><\r> |
| --- | --- |
| Description: | This packet is send from device to server, send by interval set to device. In default, our device will send this packet every 30s after login. About the <data> description, please see Appendix 1 Positon data. |
| Example: | 29 29 80 00 28 0A 81 85 0A 12 03 10 09 57 50 00 52 81 37 00 61 19 08 00 00 02 32 F8 48 FF BB FF FF 00 00 00 1E 00 00 00 00 00 00 ED 0D |
| Note: | The data between <ID> and <Checksum> is Position data. |

## Blind Area Data re-Upload – 0x8E

| Command: | ))<0x8E><L><ID><data><Checksum><\r> |
| --- | --- |
| Description: | This packet is send from device to server, send by interval set to device. In default, our device will send this packet every 30s after login. About the <data> description, please see Appendix 1 Positon data. |
| Example: | 29 29 8E 00 28 0F 80 81 5A 12 12 18 20 31 16 02 23 18 46 11 40 22 70 00 72 02 62 FB 00 07 7C 7F BF 56 00 00 1E 3C 32 00 00 00 00 85 0D |
| Note: | The data between <ID> and <Checksum> is Position data. |

## Over speed Alarm Set – 0x3F

| Command: | ))<0x3F><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This packet sent form server to device. It set a speed limit to device, when device speed up to or bigger than the limit, device will give a over speed alarm. About the alarm content, please see the Alarm Info. The unit for <parameter> is km/h |
| Example: | 29 29 3F 00 07 0A 9F 95 38 32 Checksum(Xor) 0D |
| Note: | In this command, the <parameter> is 0x32=50, means the speed limit is 50km/h. |

## Movement Alarm – 0xC7

| Command: | ))<0xC7><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command is send from server to device, to set a circle area. When device go out of the circle, it will give a move alarm. The unit of <parameter> is 10 meters. The reply for command, please see Set Reply. |
| Example: | 29 29 C7 00 07 0A 9F 95 38 0F F7 0D |
| Note: | In this example, the parameter is 0F=15, so the radum is 150 meter. |

## Stop Engine (Remote Immobilize) – 0x39

| Command: | ))<0x39><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command send from server to device, when receive this command , device will give output control signal, normally we use it control vehicle oil pump, user’s can use it control other part of vehicle by this control output. The reply for this command, please see Set Reply. Command for recover please sees Recover Engine. |
| Example: | 29 29 39 00 06 0A 9F 95 38 07 0D |
| Note: | This command will have physical operations to user’s Car, please make two confirmation for this function in software, to make sure operations to car is safe. |

## Recover Engine (Disable Immobilize) – 0x38

| Command: | ))<0x38><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command is undo action for command Stop Engine. Device gives Set Reply after this command. |
| Example: | 29 29 38 00 06 0A 9F 95 38 06 0D |
| Note: | This command will cancel the stop engine status in position data, and write 0x38 in V8 part. |

## Listen in (Voice Monitoring) – 0x3E

| Command: | ))<0x3E><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command will request device make a one-way telephone to the number in <parameter> part. Parameter part use compressed BCD code. Device gives Set Reply for this command. |
| Example: | 29 29 3E 00 0D 0A 9F 95 38 07 55 61 52 45 81 FF 51 0D |
| Note: | For <parameter> part, it use Compressed BCD code to express telephone number, if the length of the telephone number is a odd number, need add F in the end, for example: number 13590251897 should express as: 13 59 02 51 89 7F; if the length of the telephone number is a even number, need add FF in the end, for example: number 075561524581 should be expressed as 07 55 61 52 45 81 FF. |

## Set Geo-Fencing – 0x46

| Command: | ))<0x46><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command will set Geo-Fences to device (only supported by 103/108/269). Device checks if Geo-Fence alarm triggered. Device give Set Reply to this command. |
| Example: | 29 29 46 00 3D 0A 9F 95 38 03 02 23 39 95 11 35 88 69 02 23 21 23 11 40 15 84 00 00 02 23 18 95 11 40 27 79 02 23 09 81 11 40 43 03 01 00 02 23 28 84 11 40 36 03 02 23 23 51 11 40 56 22 02 00 6C 0D |
| Note: | In example, it have set 3 Geo-fences, the meaning of the parameters shows below: 03 //Geo-fences quantities, in this example have 3 Areas. 02 23 39 95 //1st Geo-Fence. Latitude for upper left corner It is 22°33.995′ 11 35 88 69 //1st Geo-Fence. Longitude for upper left corner It is 113°58.869′ 02 23 21 23 //1st Geo-Fence. Latitude for lower right corner It is 22°32.123′ 11 40 15 84 //1st Geo-Fence. Longitude for lower right corner It is 114°1.584′ 00 //Geo-Fence number, 1st one start with 00 00 //Alarm type, 0x00 means in region alarm 0x01 means out region alarm 0x02 means alarm when both in/out alarm 0x03 means forbidden alarm 02 23 18 95 //2nd Geo-Fence. Latitude for upper left corner 11 40 27 79 //2nd Geo-Fence. Longitude for upper left corner 02 23 09 81 //2nd Geo-Fence. Latitude for lower right corner 11 40 43 03 //2nd Geo-Fence. Longitude for lower right corner 01 //Geo-Fence number, 2nd one. 00 //Alarm types. 0x00 means in region alarm. 02 23 28 84 //3rd Geo-Fence. Latitude for upper left corner 11 40 36 03 //3rd Geo-Fence. Longitude for upper left corner 02 23 23 51 //3rd Geo-Fence. Latitude for lower right corner 11 40 56 22 //3rd Geo-Fence. Longitude for lower right corner 02 //Geo-Fence number, 3rd one. 00 //Alarm type, 0x00 means in region alarm. |

## Cancel Geo-Fencing – 0x47

| Command: | ))<0x47><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device. It will delete all the Geo-Fences set in device. |
| Example: | 29 29 47 00 06 0A 9F 95 38 79 0D |
| Note: | Device sends Set Reply to server after this command. |

## Inquiry GEO Fencing – 0x48

| Command: | ))<0x48><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device. Device will reply Geo-Fences set in device. |
| Example: | 29 29 48 00 06 0A 9F 95 38 76 0D |
| Note: | Device reply 0x94 packet. |

## Send Dispatch Info – 0x3A

| Command: | ))<0x3A><L><ID><info content><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device. It used in device which have dispatch LCD installed, control Center send this command to device to inform driver next mission or else. Device will show it in LCD when receive it from control center. Device will give Set Reply after receive this command. |
| Example: | NO.1----- 29 29 3A 00 12 0A 9F 95 38 68 65 6C 6C 6F 2C 77 6F 72 6C 64 21 1D 0D NO.2------ 29 29 3A 00 0C 0A A4 83 11 2A 32 36 39 23 43 7D 0D |
| Note: | NO.1----- we send “hello, world!” to device(10312156), the parameter is the ASCII code of “hello, world!” in hex. NO.2------means send”*269#C” to device(10360317), the parameter is the ASCII code of “*269#C” in hex. |

## Delete Mileage – 0x66

| Command: | ))<0x66><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device. It will clear the mileage calculated by your device, and restart from 0. |
| Example: | 29 29 66 00 06 0A 9F 95 38 58 0D |
| Note: | Device will give Set Reply after this command. |

## Alarm Info – 0x82

| Command: | ))<0x82><L><ID><Data><Checksum><\r> |
| --- | --- |
| Description: | This command sends from device to server. It is alarm data, this packet need deal with in the first class. This command words is for important and emergency situation. |
| Example: | 29 29 82 00 23 0A A2 CC 39 12 05 03 05 05 22 02 85 94 79 03 10 95 50 00 80 02 07 84 00 00 00 02 00 00 00 00 00 00 75 0D |
| Note: | About the instruction for the <data> the parse shows below：12 05 03 05 05 22 02 85 94 79 03 10 95 50 00 80 02 07 84 00 00 this 21 byte data check in section Appendix 1 Position Data please; 00 02 00 this part is alarm status data, please check the instruction in section Appendix 2 Alarm Data; for this example, it is over speed alarm happens. 00 00 00 00 00 this part is alarm parameters data, please check the instruction in section Appendix 2 Alarm Data; |

## Cancel Alarm – 0x37

| Command: | ))<0x37><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command is sends from server to device, it will clear the alarm status in device, and the alarm status will clear to 0. And it only be reset when alarm status happens again. Device will reply Set Reply after this command. |
| Example: | 29 29 37 00 06 0A 9F 95 38 09 0D |
| Note: | This command only clear the alarm status happens before, if the alarm status is continuously, like overspeed alarm, if you speed not low down, it will keep sending this overspeed alarm. |

## Set GPRS Parameters – 0xC1

| Command: | ))<0xC1><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device. It will change current APN, APN User，APN Password set in device. Devices reply Set Reply after this command. |
| Example: | 29 29 C1 00 17 0A 9F 95 38 22 43 4D 4E 45 54 2C 67 70 72 73 2C 67 70 72 73 22 BF 0D |
| Note: | The parameter part is in ASCII code, in example: 22 43 4D 4E 45 54 2C 67 70 72 73 2C 67 70 72 73 22 Means:”CMNET,gprs,gprs” |

## Set Server IP and Port In Device – 0x69

| Command: | ))<0x69><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device. It will change current server IP and PORT set in device. Device reply Set Reply after this command. |
| Example: | 29 29 69 00 1C 0A 9F 95 38 22 31 32 31 2E 30 33 37 2E 30 35 39 2E 32 31 37 22 2C 37 37 37 37 41 0D |
| Note: | Parameter part is in ASCII code. In the example, the <parameter> part: 22 31 32 31 2E 30 33 37 2E 30 35 39 2E 32 31 37 22 2C 37 37 37 37 means: ”121.037.059.217”,7777 Remark: The IP data head and tail contain a double quotation marks. |

## Set Reply – 0x85

| Command: | ))<0x85><L><ID><Data><Checksum><\r> |
| --- | --- |
| Description: | This package sends from device to server, for reply most of the commands send from server to device. It will put the command words of packet it reply for in the last byte of <data> part. The detail description of <Data> part please find in Appendix 1 Position Data. |
| Example: | 29 29 85 00 08 1D D0 8C 22 12 03 12 17 40 26 02 65 45 71 03 12 54 17 00 00 00 00 F8 19 C8 39 FF FF 1D 00 00 1E 00 50 00 00 00 3A F9 0D |
| Note: | In this example, the last byte of <Data> part is 0x3A, so it is a reply packet for Send Dispatch Info. |

## Illegal Ignition Set -0xC6

| Command: | ))<0xC6><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device. It will define time period when vehicle can’t be started, if detect the device started in forbidden time period; device will give an alarm Illegal Ignition. Every device can set 10 time period in device. Device give Set Reply after receive this command. |
| Example: | 29 29 C6 00 13 0A 9F 95 38 01 0C 01 01 01 14 00 0C 0C 1F 00 17 3B C0 0D |
| Note: | In this example, the <parameter> part should be parse like this: 01 // the serial number of time period, from 0x01~0x0A. 0C 01 01 01 14 00 // this is start time. First byte 0x0C is year, system need add 2000 to this part, so the value range of year is 2000~2255, so 0x0C means 2012; second byte 0x00 is for month, value range is 01~0C, in example 0x01 means JAN; the third byte 0x01 is day, value range is 01~1F; the fourth byte 0x01 means week, value range is 1~7, 7 means Sunday; the fifth byte means hour, value range is 0~17h, 0x14 means 20:00; the last byte is min, value range is 0~3B. 0C 0C 1F 06 17 3B // this is end time, define is the same with start time. When set year month week day set to 0, means device do not check that part, just check time. |

## Device Factory Reset – 0xC3

| Command: | ))<0xC3><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device, command will recover all the settings except IP/PORT/APN/APN user/APN password to device include time interval, over speed setting, and mileage and so on to factory set. Device give Set Reply after receive this command. |
| Example: | 29 29 C3 00 06 0A 9F 95 38 FD 0D |
| Note: | Attention, this command will clear all the alarm settings and mileage count data, be careful when send this command. |

## Set Vehicle Idle Alarm – 0x4B

| Command: | ))<0x4B><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device, it will set a time during to device, if device detect that the vehicle is engine on and speed<10km/h for the time during set or longer, device will send a vehicle Idle alarm. You can find the alarm packet in Alarm Info, and find alarm status instruct in Appendix 2 Alarm data. |
| Example: | 29 29 4B 00 07 0A 9F 95 38 14 60 0D |
| Note: | The parameter unit is minute, in this example, 0x14 means 20 minutes. |

## Check Device Version – 0x3D

| Command: | ))<0x3D><L><ID><Checksum><\r> |
| --- | --- |
| Description: | This command sends from server to device, to check the firmware version of device. Device will reply 0x84 to this command. |
| Example: | 29 29 3D 00 06 0A 9F 95 38 03 0D |
| Note: | This command is for firmware version check only. For some basic mode of tracker like 101A, old version of firmware may do not support this command. |

## Check Device Version Reply – 0x84

| Command: | ))<0x84><L><ID><Data><Checksum><\r> |
| --- | --- |
| Description: | This command is reply for firmware check, send from device to server. <Data> part is firmware version of this device, in ASCII code. |
| Example: | 29 29 84 00 31 0A A2 8A 07 54 33 36 30 2D 31 30 33 41 57 28 47 54 4D 29 56 32 2E 30 31 20 40 20 53 65 70 20 32 33 20 32 30 31 31 20 31 32 3A 34 38 3A 31 38 CF 0D |
| Note: | In this example, the firmware version of this device is : T360-103AW(GTM)V2.01 @ Sept. 23 2011 12:48:18 |

## Get Current Address – 0xA8

| Command: | ))<0xA8><L><ID><Data><Checksum><\r> |
| --- | --- |
| Description: | This command is used when customer need to get detail address, like building, street, city. With our 101A and 101E device. Customer can send SMS command: AS1234WHERE1# to device, device sends this command to server to require address from server. About the reply please see Get Current Address Reply. |
| Example: | 29 29 A8 00 28 0A A2 8A 07 12 03 12 17 40 26 02 65 45 71 03 12 54 17 00 00 00 00 F8 19 C8 39 FF FF 1D 00 00 1E 00 50 00 00 00 00 95 0D |
| Note: | The <Data> part is current position data, you can check the instruct about it in Appendix1 Position Data. |

## Get Current Address Reply – 0x72

| Command: | ))<0x72><L><ID><Data><Checksum><\r> |
| --- | --- |
| Description: | This command is reply for address requirement 0xA8, the address is <Data> part, coding in Unicode. Device will get the <Data> part of address and send SMS to the customers mobile phone who require the address. |
| Example: | 29 29 72 00 5B 0A A2 8A 07 00 00 54 00 65 00 6D 00 61 00 20 00 2D 00 20 00 53 00 61 00 64 00 66 00 61 00 2C 00 20 00 53 00 61 00 6C 00 61 00 6D 00 6F 00 75 00 6E 00 2C 00 20 00 4D 00 61 00 72 00 6B 00 61 00 7A 00 20 00 54 00 61 00 6D 00 61 00 2C 00 20 7D 22 54 C8 67 70 57 C3 53 CA 89 0D |
| Note: | In this example, the address get from server is : Tema – Sadfa, Salamoun, Markaz Tema, 索哈杰埃及 |

## Remote Update Firmware – 0xD2

| Command: | ))<0xD2><L><ID><Data><Checksum><\r> |
| --- | --- |
| Description: | This command is used for update firmware by GPRS. It is used when device have new function or bugs need be fixed remotely. When received this command device will check and connect to certain update TFTP sever to download the new firmware file then update. Device will give Set Reply after receive this command. |
| Example: | 29 29 D2 00 2D 0A A2 8A 07 01 31 32 31 2E 30 33 37 2E 30 35 39 2E 32 31 37 22 C1 6E 65 77 2E 62 69 6E 00 00 00 00 00 00 00 00 00 00 00 00 00 00 2F 0D |
| Note: | The <data> part in the example parsed as below: 01 //Update type, if it is 0x00, then cancel update; when it is 0x01 then force update. 31 32 31 2E 30 33 37 2E 30 35 39 2E 32 31 37 //update server IP: 121.037.059.217; 15 bytes of data in ASCII code. 22 C1 //Update server port:8897 ; data is in hex; 6E 65 77 2E 62 69 6E 00 00 00 00 00 00 00 00 00 00 00 00 00 00 //file names: new.bin, 21bytes (add 00 when less than 21 bytes), in ascii code. |

## Remote Update Firmware Result – 0xA5

| Command: | ))<0xA5><L><ID><Data><Checksum><\r> |
| --- | --- |
| Description: | This command is uploading by device. After firmware update, device will upload this packet to software, to show update successful or not. The flag for successful is in the last byte of <Data> part, when it is 0x00, it means update successful, if it is not 0x00, means update failed. |
| Example: | 29 29 A5 00 28 0A A2 8A 07 12 03 12 17 40 26 02 65 45 71 03 12 54 17 00 00 00 00 F8 19 C8 39 FF FF 1D 00 00 1E 00 50 00 00 00 00 98 0D |
| Note: | The <Data > part is position data, the last byte of it is 0x00, it is V8 of position data. In this command it means update successful. |

## Camera Related Command – 0x28/0x65/0x26/0xAB

Please refer to file <<Amwell Take Photo Protocol V1.0>>.

## Temperature alarm set – 0x71

| Command: | ))<0x71><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command is used to configure the temperature abnormal alarm parameter. |
| Example: | 29 29 71 00 0B 0A 9F 95 38 Byte1 Byte2 Byte3 Byte4 Byte5 Xor 0D |
| Note: | 1). Byte1: Temperature abnormal alarm enable/disable mark byte. Example, Byte1=0x00 disable, Byte1=0x01 enable. Byte2Byte3: Temperature upper limit value, unit 1/10℃. Signed short integer, the MSB used to be a sign bit. Example, Byte2Byte3=0x0123=291, so it means the upper temperature limit is 291/10=29.1℃. Byte4 Byte5: Temperature drop limit value, unit 1/10℃. Signed short integer, the MSB used to be a sign bit. Example, Byte4Byte5=0x8123=-291, so it means the drop temperature limit is -291/10=-29.1℃. 2). Device reply by command 0x85, and set data field V8=0x71 to indicate ACK. Remark: Only connected temperature sensor will have this function. Default enable. |

## Peripheral data info upload –0xA3

| Command: | ))<0XA3><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This packet will be send from device to server by set interval time to replace the command packet 0x80 when device has connected peripheral unit like fuel tank sensor, temperature sensor etc. Anyway, this packet equal 0x80 packet, but added peripheral sensor data. |
| Example: | 29 29 A3 L 0A 9F 95 38 Location_info Peripheral_data Xor 0D |
| Note: | 1). Location_info: Please see Appendix 1 Positon data. Peripheral_data: see Appendix 3 peripheral data. |

## Fuel level sensor Parameter Set – 0xD3

| Command: | ))<0xD3><L><ID><Data><Checksum><\r> |
| --- | --- |
| Description: | This command is used to configure the fuel sensor parameter to the device from server. |
| Example: | 29 29 D3 00 0B 0A A2 8A 07 Byte1 Byte2 Byte3 Byte4 Byte5 Xor 0D |
| Note: | 1). Byte1 Byte2: used to indicate the fuel sensor voltage when fuel tank is empty. Byte3 Byte4: used to indicate the fuel sensor voltage when fuel tank is full. Byte5: used to indicate the fuel sensor type, 0x00 means using vehicle original fuel sensor, 0x01 means using external fuel sensor. 2). Device reply by command 0x85, and set data field V8=0xD3 to indicate ACK. Remark: Only T360-108 device support. Default enable. |

## Real-time tracking by Interval – 0xD4

| Command: | ))<0xD4><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command is used to configure the real-time tracking parameter. |
| Example: | 29 29 D4 00 0A 0A 9F 95 38 Byte1 Byte2 Byte3 Byte4 Xor 0D |
| Note: | 1). Byte1Byte2: used to configure the real-time tracking location info upload times. Example, Byte1Byte2=0x0064=100, means that device will upload location point info by Byte3Byte4 interval and up to 100 times, this command will be lose efficacy. Expressly, Byte1Byte2=0x0000 means stop the real-time tracking, and Byte3Byte4=0xFFFF means current real-time tracking is always effective until be changed. Byte3Byte4: used to configure the real-time tracking interval. Example, Byte3Byte4=0x001E=30, means that the real-time tracking interval is 30sec. 2). Device reply by command 0x85, and set data field V8=0xD4 to indicate ACK. |

## Tracking compensation by angle – 0xD9

| Command: | ))<0xD9><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command is used to configure the tracking compensation of angle parameter. When vehicle make a turn more than this angle value, device will upload one location info. |
| Example: | 29 29 D9 00 D7 0A 9F 95 38 Byte1 Xor 0D |
| Note: | 1). Byte1 : The angle value configured, the unit is degree. Example, Byte1=0x1E(30 degree), means that when vehicle make a turn more than 30degree, device will upload one location info. 2). Device reply by command 0x85, and set data field V8=0xD9 to indicate ACK. |

## Fatigue driving alarm set – 0xDA

| Command: | ))<0xDA><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command is used to configure the fatigue driving alarm parameter. |
| Example: | 29 29 DA 00 0A 0A 9F 95 38 Byte1 Byte2 Byte3 Byte4 Xor 0D |
| Note: | 1). Byte1 Byte2: Maximum driving time limit, unit minute. Example Byte1Byte2=0x0102 (0x0102=258), means the max driving time limit is 258minutes. Specially, when Byte1Byte2=0x0000, means cancel checking this alarm. Byte3 Byte4: The minimum driver to need a reset time, unit minute. Example Byte3Byte4=0x0014 (0x0014=20), means that the minimum driver to need a reset time is 20minutes. 2). Device reply by command 0x85, and set data field V8=0xDA to indicate ACK. |

## Over Speed alarm and trig-time set – 0xDB

| Command: | ))<0xDB><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command is used to configure the over speed and trig-time limit val. |
| Example: | 29 29 DB 00 09 0A 9F 95 38 Byte1 Byte2 Byte3 Xor 0D |
| Note: | 1). Byte1: Over speed value limit, unit km/h. Example Byte1=0x64 (0x64=100), means the speed limit is 100km/h. Specially, Byte1=0x00 means cancel over speed alarm checking. Byte2 Byte3: Over speed trig-time limit, unit sec. Example Byte2Byte3=0x000A (0x000A =10), means that the over speed limit trig-time is 10sec. 2). Device reply by command 0x85, and set data field V8=0xDB to indicate ACK. Only T360-108 device support this command. |

## Device odometer value set – 0xDC

| Command: | ))<0xDC><L><ID><Parameter><Checksum><\r> |
| --- | --- |
| Description: | This command is used to set the device odometer value. |
| Example: | 29 29 DC 00 08 0A 9F 95 38 Byte1 Byte2 Xor 0D |
| Note: | 1). Byte1 Byte2: The device odometer value setting, unit Km, unsigned short integer. Example Byte1Byte2=0x0123=291, means that setting device odometer value to 291Km. 2). Device reply by command 0x85, and set data field V8=0xDC to indicate ACK. |

# Appendix 1 Position Data

There have 2 kinds of Position data, one is for normal position data, and another is for alarm data (such as 0x82).

- ymdhms wwww jjjj ssff st lichen1 lichen2 lichen3 st1st2st3st4 v1v2v3v4v5v6v7v8

Ex: 12 03 12 17 40 26 02 65 45 71 03 12 54 17 00 00 00 00 F8 19 C8 39 FF FF 1D 00 00 1E 00 50 00 00 00 00

- ymdhms wwww jjjj ssff p load sign  (for alarm packet)

Ex: 12 05 03 05 05 22 02 85 94 79 03 10 95 50 00 80 02 07 84 00 00

From the format, we can see that these 2 kinds of data have same part: ymdhms wwww jjjj ssfffst, and different parts: lichen1 lichen2 lichen3 st1st2st3st4 v1v2v3v4v5v6v7v8 and load sign.

Will describe it in 3 parts:

- instruct for ymdhms wwww jjjj ssff

| Field Name | Length(Byte) | Content | Note |
| --- | --- | --- | --- |
| Datetime | 6 | ymdhms 12 03 12 17 40 26 | Date range: Year, 2000-2099; Month 1-12; Day, 1-31; Time range: Hour, 00-23; Min, 00-59; Second, 00-59. Hour/min/second use compressed BCD code For example: 12 03 12 17 40 26 means 2012-3-12 17:40:26 |
| Latitude | 4 | wwww 02 65 45 71 | Latitude range: 00°00.000′~89°59.999′. Use compressed BCD code, But the highest bit is sign bit. “Positive” Means “North Latitude”, “negitive” means “south latitude”; The Unit for ′ is 0.001′ For example: South latitude 30°37.901′express as: 83H, 03H, 79H, 01H 02 65 45 71 means: north latitude 26°54.571′. |
| Longitude | 4 | jjjj 03 12 54 17 | Longitude range: 000°00.000′~179°59.999′; Use compressed BCD code, the highest bit is sign bit. “Positive” Means “east longitude”, “negitive” means “West longitude”; The Unit for ′ is 0.001′ For example: west longitude 130°45.608′express as: 93H, 04H, 56H, 08H 03 12 54 17 means: 31°25.417′. |
| Speed | 2 | ss 00 80 | Speed express method: Range: 0-9999km/h Use compressed BCD code. 00 80=(00H,80H)=80km/h For example: 120km/h express as: 01H, 20H |
| Direction | 2 | ff 02 07 | Direction range: 000-359° Use compressed BCD code, true North is 0°, clockwise counting. The unit is degree. For example: 154°express as: 01H, 54H 02 07 means 207°. |

- Instruct for st lichen1 lichen2 lichen3 st1st2st3st4 v1v2v3v4v5v6v7v8

| Field Name | Length(Byte) | Content | Note |
| --- | --- | --- | --- |
| GPS/antenna /power status | 1 | st F8 | D7; 0 Unlocated; 1 Located. D6; 1 1 GPS Normal 1 0 GPS antenna short 0 1 GPS antenna cut 0 0 GPS module error. D5. D4; 1 1 main power normal 1 0 main power lose 0 1 main power too high or too low. D3. D2; D2D1D0: 010: odometer unit Km. 011: Version from V2.02. Other value reserve. normally 000. D1. D0. In the example F8 =11111000, means located/gps normal/power normal/aw protocol |
| Mileage | 3 | lichen1 lichen2 lichen3 19 C8 39 | HEX format 0x00-0XFFFFFF (16777215) meters. 19 C8 39 means 1689657m |
| Status | 4 | st1st2st3st4 FF FF 1D 00 | bit; St1; St2; St3; St4. D7; 1 acc off 0 acc on; 1 normal 0 SOS; 1 Fatigue driving 0 normal; 1 output(1) 0 no output. D6; 1 self-define(1) no alarm 0 alarm happen; 1 normal 0 over speed; 1 need reply 0x21 0 do not need reply; 1 output(2) 0 no output. D5; 1 user define (2) no alarm 0 alarm happen; 1 normal 0 stop overtime; 1 Temperature abnormal 0 normal; 1 GPRS traffic Abnormal 0 normal. D4; 1 user define (3) no alarm 0 alarm happen; 1 normal 0 out area alarm; CSQ value: 0~31; 1 have taxi meter 0 no taxi meter. D3; 1 user define (4) no alarm 0 alarm happen; 1 normal 0 in area alarm; 1 have broadcast 0 no voice broadcast. D2; 1 fuel pump not cut 0 fuel pump cut.; 1 user define (5) no alarm 0 alarm happen; 1 forbid dial out 0 allow dial out. D1; 1 not sign in 0 sign in; 1 user define (6) no alarm 0 alarm happen; 1 forbid call in 0 allow call in. D0; 1 not defencing 0 defencing; 1 user define (7) no alarm 0 alarm happen; 1 forbid call 0 allow call. FF FF 1D 00 means acc off / no self define alarm / no immobilize/ not sign in/ no defence/ no alarms/ gprs registerd/no need reply 0x21 command/ UDP mode/ csq=29/no accessories. |
| V1V2 | 2 | v1v2 | A random number, used for device remote debug. |
| V3 stop over time setting | 1 | v3 | This part is parking/stop over time setting value, in default it is 00. A random number, used for device remote debug. |
| V4 | 1 | v4 | This value is speed limit setting in device, 0x50 means the limit is 80km/h; in default it is 00. A random number, used for device remote debug. |
| V5 | 1 | v5 | A random number, used for device remote debug. |
| V6 | 1 | v6 | A random number, used for device remote debug. |
| V7 | 1 | v7 | A random number, used for device remote debug. |
| V8 command word send by control center | 1 | v8 00 | This is used when reply to command send by control center, device put the command word send by control center to this byte. |

- Instruct for  p load sign

| Field Name | Length(Byte) |  | Content |  | Note |
| --- | --- | --- | --- | --- | --- |
| GPS/antenna /power status | 1 | p 84 |  | D7; 0 Unlocated; 1 Located. D6; 1 1 GPS antenna normal 1 0 GPS antenna short circuit 0 1 GPS antenna open circuit 0 0 GPS module error. D5. D4; 1 1 power normal 1 0 power off 0 1 power unnormal(too high/low). D3. D2; 1 1 1 mileage unit is m 0 0 0 103/269 devices,mileage unit is m 0 1 0 mileage unit is Km 0 1 1 mileage unit is Km. D1. D0. In the example 84 =10000100, means located/do not use DGPS/4 satellites visiable |
| Empty-loaded status | 1 |  | load |  | Use compress BCD code, 1 = load; 0=empty |
| Sign up status | 1 |  | sign |  | Use compress BCD code, 1=sign in; 0=sign out |

# Appendix 2 Alarm Data

In alarm info packet, there have 8 bytes of data to describe the alarm status and alarm parameters. For example: 29 29 82 00 23 0A A2 CC 39 12 05 03 05 05 22 02 85 94 79 03 10 95 50 00 80 02 07 84 00 00 00 02 00 00 00 00 00 00 75 0D. The part 00 02 00 00 00 00 00 00 is what we will describe in this part.

About the 8 bytes of data, first 3 bytes for alarm status and 5 bytes for alarm parameters.

- Instruct for alarm status 00 02 00

|  | 1st byte | 2nd byte | 3rd byte |  |
| --- | --- | --- | --- | --- |
| D7 | In area alarm | Door open alarm (User define(3) alarm) | Idle alarm |  |
| D6 | Out area alarm | Tow alarm | Fatigue alarm |  |
| D5 | Offset router alarm | G Sensor alarm (see 4th byte in alarm parameters ) | Parking status |  |
| D4 | Low power alarm | User define (4) alarm | Remove Alarm |  |
| D3 | User define(5) alarm | Power cut alarm | GPRS flow over limit alarm |  |
| D2 | User define(2) alarm | Stop over time alarm | Temperature abnormal |  |
| D1 | User define(1) alarm | Over speed alarm | Reserved |  |
| D0 | Illegal ignition alarm | Panic alarm | Reserved |  |

1 means alarm happen; 0 means no alarm. So 00H 02H 00H = 00000000B 00000010B 00000000B means over speed alarm; 00H 00H 04H = 00000000B 00000000B 00000100B means Temperature Abnormal.

- Instruct for alarm parameters 00 00 00 00 00

|  | 1st byte | 2nd byte | 3rd byte | 4th byte | 5th byte |
| --- | --- | --- | --- | --- | --- |
| D7 | Geo-Fence Number of GEO Fence alarm | Reserved | Route number of Offset route alarm | Crash alarm | Reserved |
| D6 |  | Reserved |  | Turnover | Reserved |
| D5 |  | Reserved |  | Harsh Acceleration | Reserved |
| D4 |  | Reserved |  | Harsh deceleration | Reserved |
| D3 |  | Reserved |  | Shake alarm (detect when acc off) | Reserved |
| D2 |  | User define (7) alarm |  | Reserved | Reserved |
| D1 |  | User define (6) alarm |  | Fuel stolen alarm | Reserved |
| D0 |  | ACC status (0 on ;1 off) |  | Low fuel alarm | Reserved |

| Sign up status | 1 | sign | Use compress BCD code, 1=sign in; 0=sign out |
| --- | --- | --- | --- |

# Appendix 3 peripheral data

Peripheral data structure:

| Field Name | Length(Byte) | Description |
| --- | --- | --- |
| All KLV data length | 2 | The data length of all KLV list. |
| KLV data list | N | KLV list, see Peripheral data KLV list as follow. |

Peripheral data KLV list:

| K(key) | L(length) | V(value) |
| --- | --- | --- |
| 0x01 | N | Reserve. |
| 0x02 | N | Reserve. |
| 0x03 | 11 | RFID action info: Byte1(0:sign off, 1:sign on) + Byte2~Byte11(ascii string, filling 0x00 to the remaining bytes when less than 10 bytes). |
| 0x04 | N | Reserve. |
| 0x05 | N | Reserve. |
| 0x06 | N | Reserve. |
| 0x07 | N | Reserve. |
| 0x08 | N | Reserve. |
| 0x09 | 3 |  |
| 0x0A | N | Reserve. |
| 0x0B | N | Reserve. |
| 0x0C | N | Reserve. |
| 0x0D | 2 | Temperature value. Bit15: signed mark, Bit0-14: value. The unit is 0.1℃. For example: 0x000A = 1℃, 0x800A=-1℃. |
| 0x0E | 1 | Fuel tank level percent: 0-100%. |
| 0x0F | N | Reserve. |
| 0x10 | N | Reserve. |
| 0x11 | N | Reserve. |
| 0x16 | N+1 | MIFARE Card/iButton: Format: Sign status(1BYTE)+CARD NO.(n BYTE,ASCII) Explain: 0x00 sign in start+CARD NO.(n BYTE,ASCII) 0x01 sign in status+CARD NO.(n BYTE,ASCII) 0X02 sign out+CARD NO.(n BYTE,ASCII) |

KLV structure frame:

K(1byte): the key id of the data part.

L(1byte): the length of the data part of V.

V(nbyte): the value of the data part.

For example, Temperature sensor KLV: 0x0D 0x02 0x00 0x0A, means 1℃.